Readme for TCPSocketClient in Linux

Purpose:

This document describes the purpose, requirements, installation and operation of the TCPSocketClient application. TCPSocketClient is a host application which is used for controlling and monitoring the Genesys series of Lambda power supply using TCP communication system. The communication port is 8003.

Requirements:

- Host PC with Linux Operating System.

- Ethernet connectivity.

Installation:

- After untaring the “GEN LAN TCP Socket LinuxC v01.tar.gz”, the executable “TCPSocketClient” can be found from the directory “/TCPSocketClient/”.

- Run the file “TCPSocketClient” as “./TCPSocketClient” using the terminal.

Operation:

- After starting the application, the user can enter the IP Address or hostname of the power supply to be connected with the client.

- Then enter the SCPI command or query.

- If the Host does not exist, socket creation or connection fails a network related error is displayed.

- If Web Admin is logged in or any other client is connected in “One Client Only” access mode, then TCP connection will be refused and the network related error code will be displayed.

- If the SCPI command is a command, the application just send the command to the power supply.

- If the SCPI command is a query then the application shows the response.

- If the query is correct then the proper reply is obtained. Otherwise it waits for 10 seconds and a network related timeout error is displayed. The system error can be obtained by sending a SCPI command “SYST:ERR?”.

- After closing the socket, client application gives a “success” message.

- The user can again give SCPI command to continue or “EXIT” command to close the application.