Readme for UDPSocketClient in Linux

Purpose:

This document describes the purpose, requirements, installation and operation of the UDPSocketClient application. UDPSocketClient is a host application which is used for controlling and monitoring the Genesys series of Lambda power supply using UDP communication system. The communication port is 8005.

Requirements:

- Host PC with Linux Operating System.

- Ethernet connectivity.

Installation:

- After untaring the “GEN LAN UDP Socket LinuxC v01.tar.gz”, the executable “UDPSocketClient” can be found from the directory “/UDPSocketClient/”.

- Run the file “UDPSocketClient” as “./UDPSocketClient” using the terminal.

Operation:

- After starting the application, the user can enter the IP Address or hostname of the power supply to be connected with the client.

- Then enter the SCPI command or query.

- If the Host does not exist, socket creation or sending fails a network related error is displayed.

- If Web Admin is logged in, the UDP Socket will be disabled and error message “ERROR: Port disabled because Admin is logged onto the web page” is displayed.

- If the LAN card controller access is “One Client Only”, then UDP Socket will be disabled and error message “ERROR: Socket blocked by single-client setting” is displayed.

- If the SCPI command is a command, the application just send the command to the power supply.

- If the SCPI command is a query then the application shows the response.

- If the query is correct then the proper reply is obtained. Otherwise it waits for 10 seconds and a network related timeout error is displayed. The system error can be obtained by sending a SCPI command “SYST:ERR?”.

- After closing the socket, client application gives a “success” message.

- The user can again give SCPI command to continue or “EXIT” command to close the application.