Readme for TCPSocketClient in Windows

Purpose:

This document describes the purpose, requirements, installation and operation of the TCPSocketClient application. TCPSocketClient is a host application which is used for controlling and monitoring the Genesys series of Lambda Power supply using TCP communication system. The communication port is 8003.

Requirements:

- Host PC with Windows operating system.

- Ethernet connectivity.

Installation:

- After unzipping the “GEN LAN TCP Socket Cpp VS2005 v01.zip”, “TCPSocketClient.exe” can be found from the directory “\TCPSocketClient\Release\”.

- Run the file “TCPSocketClient.exe”.

Operation:

- After starting the application, the user can enter the IP Address or hostname of the power supply to be connected with the client.

- Then enter the SCPI command or query in the SCPI command field.

- Press the Run button.

- If web admin is logged in or any other client is connected in “One Client Only” access mode, then TCP connection will be refused and the network related error code will be displayed in the status field.

- If the hostname is given and it does not exist, socket creation or connection fails, a network related error is displayed in the status field.

- If the connection is success, the command is send to the power supply.

- If the SCPI command is a command, the application just send the command to the power supply.

- If the SCPI command is a query then the application shows the response and status.

- If the query is correct then the proper reply is obtained in the response field and status field shows “success”. Otherwise it waits for 10sec and a network related timeout error is displayed in the status field. The system error can be obtained by sending a SCPI command “SYST:ERR?”.

- After closing the socket, client application gives a “success” status field.

- The user can again give SCPI command to continue or click the Quit button to close the application.