Read me for UDPSocketClient in Windows

Purpose:

This document describes the purpose, requirements, installation and operation of the UDPSocketClient application. UDPSocketClient is a host application which is used for controlling and monitoring the Genesys series of Lambda Power supply using UDP communication system. The communication port is 8005.

Requirements:

- Host PC with Windows operating system.

- Ethernet connectivity.

Installation:

- After unzipping the “GEN LAN UDP Socket Cpp VS6.0 v01.zip”, “UDPSocketClient.exe” can be found from the directory “\UDPSocketClient\Release\”.

- Run the file “UDPSocketClient.exe”.

Operation:

- After starting the application, the user can enter the IP Address or hostname of the power supply to be monitored and controlled.

- Then enter the SCPI command or query in the SCPI command field.

- Press the Run button.

- If web admin is logged in, UDP socket will be disabled and the error message “ERROR: Port disabled because Admin is logged onto the web page.” is displayed in the response field.

- If the LAN card controller access is “One Client Only”, then UDP Socket will be disabled and the error message "ERROR: Socket blocked by single-client setting" is displayed in the response field.

- If the hostname is given and it does not exist or socket creation fails, a network related error is displayed in the status field.

- Otherwise, the command is send to the power supply.

- If the SCPI command is a command, the application just send the command to the power supply.

- If the SCPI command is a query, then the application shows the response and status.

- If the query is correct then the proper reply is obtained in the response field and status field shows “success”. Otherwise it waits for 10sec and a network related timeout error is displayed in the status field. The system error can be obtained by sending a SCPI command “SYST:ERR?”.

- After closing the socket, client application gives a “success” status field.

- The user can again give SCPI command to continue or click the Quit button to close the application.